# Liability Allocation in Accidents Involving

# Vehicles with Advanced Driver-Assistance  Systems(ADAS)in Shared Human-Machine Driving Contexts

Introduction

Advanced Driver-Assistance Systems(ADAS)represent a critical evolution in automotive technology,bridging traditional human-driven vehicles and fully autonomous systems.

These systems,typically classified under SAE Levels 1-3,involve shared control where the human driver remains responsible for monitoring and intervening as needed.However,

accidents in these contexts raise complex questions about liability allocation,as

responsibility  may  span  drivers,manufacturers,software  developers,and   regulators.This report examines the technical foundations of ADAS,reviews prevailing legal frameworks,  analyzes pertinent case law,delineates responsibility boundaries,and offers regulatory     recommendations.Drawing on current developments as of December 2025,it aims to

provide a balanced perspective on mitigating risks while fostering innovation.

## 1.Key Technical Principles of ADAS

ADAS encompass features like adaptive cruise control,lane-keeping assistance,automatic emergency braking,and blind-spot detection,designed to enhance safety by augmenting   human capabilities.Understanding their technical principles is essential for assessing

liability,as limitations can contribute to accidents in shared driving scenarios.

Sensor Limitations

ADAS rely on sensors such as radar,LiDAR,cameras,and ultrasonic devices to perceive

the environment.Radar excels in detecting speed and distance but struggles with non-

metallic objects or in heavy rain.LiDAR provides high-resolution 3D mapping but is

impaired by fog,snow,or dust.Cameras offer visual recognition but are limited by low light, glare,or occlusion (e.g.,blocked views by vehicles or debris).These constraints can lead to false  positives(unnecessary  interventions)or  false  negatives(missed  hazards),particularly  in adverse weather or complex urban settings.

Operational  Design  Domain(ODD)

The ODD defines the specific conditions under which an ADAS is designed to operate

safely,including  road  types(e.g.,highways  vs.city  streets),weather,time  of  day,and

speed limits.For instance,many Level 2 systems like Tesla's Autopilot are optimized for

controlled highways but may falter in construction zones or unmarked roads.Exceeding the ODD can shift liability,as systems are not intended for universal use.

Human-Machine Interface(HMI)

The HMI facilitates interaction between the driver and ADAS through visual,auditory,or    haptic alerts(e.g.,steering wheel vibrations for lane departure warnings).Effective  HMls  issue timely take-over requests when the system detects limitations,giving drivers 3-10   seconds to respond based on SAE guidelines.However,poor design-such as ambiguous warnings or over-reliance induced by marketing-can lead to "automation complacency,"  where drivers disengage mentally,exacerbating accident risks.

To illustrate the progression of autonomy:

SAE  Levels of Driving AutomationTM  Refined for Clarity and...

## 2.Existing Legal Frameworks

Legal frameworks for ADAS liability blend traditional tort principles with emerging

regulations tailored to automated technologies.These address product defects,driverr negligence,and  compliance  standards,varying  by  jurisdiction.

Product       Liability

Under product liability law,manufacturers can be held strictly liable for defects in design, manufacturing,or warnings that cause harm.In the EU,the revised Product Liability

Directive  (EU)2024/2853  expands  "product"to  include  software,Al  systems,and  over-     the-air(OTA)updates,deeming cybersecurity vulnerabilities or  update failures  as defects. This subjects AV and ADAS producers to liability for post-sale changes,with a broader

range  of  parties(e.g.,importers,platforms)potentially  accountable.In  the   US,state  laws apply strict liability via doctrines like the Restatement(Third)of Torts,recognizing Al

software as a "product"and holding manufacturers liable for algorithmic errors or data processing failures.

Negligence

Negligence claims require proving a breach of duty causing harm.Drivers owe a duty of

reasonable care,including monitoring ADAS and intervening appropriately.Manufacturers  may be negligent for inadequate testing or misleading marketing that overstates system     capabilities.In shared contexts,courts assess whether the human or machine held primary control at the time of the incident.

Regulatory         Standards

The US National Highway Traffic Safety Administration(NHTSA)mandates crash reporting for ADAS-equipped vehicles via the Standing General Order 2021-01(amended 2025),

requiring data submission on incidents involving Level 2 systems.NHTSA's ADS-Equipped   Vehicle Safety,Transparency,and Evaluation Program(proposed January 2025)establishes voluntary oversight frameworks.In the  EU,UNECE  regulations(e.g.,WP.29)set  standards   for ADAS certification,while the Data Act complements liability rules by governing data

access for investigations.These frameworks aim to balance innovation with safety but often lack specificity on shared liability.

## 3.Analysis  of  Relevant  Case  Law

Case law on ADAS accidents is evolving,with real cases highlighting challenges like defect attribution,marketing misrepresentation,and shared fault.Below are key examples from     2020-2025,illustrating   legal   hurdles.

Real     Cases:Tesla     Autopilot     Incidents

In Benavides v.Tesla,Inc.(S.D.Fla.2025),a jury awarded $242.57 million after a Tesla

Model S on Autopilot fatally struck a pedestrian in 2019.The jury allocated 33%fault to     Tesla for software defects(failure to brake or warn)and 67%to the distracted driver.This verdict underscores marketing's role,as Tesla's promotions were deemed to encourage    over-reliance,leading to punitive damages for deterrence.

The  Walter   Huang  wrongful  death  suit(Cal.Super.Ct.2024)settled   undisclosed  after claims  of Autopilot  defects  and  misleading  statements;NTSB  found  system  failures  in collision    avoidance.Conversely,in    Molander     v.Tesla(Cal.Super.Ct.2023),the    jury

absolved  Tesla,attributing  fault  to  the  driver's  intoxication  despite  alleged  steering defects.

In  Justine  Hsu  v.Tesla(L.A.Super.Ct.2023),no  liability  was  found  for  injuries  from  an

Autopilot  curb  collision,emphasizing  clear  warnings  against  misuse  on  city  streets.A  class action on  misleading Autopilot  capabilities was dismissed  in  2023 due to arbitration

clauses.

Illustrative Challenges

These  cases  reveal  inconsistencies:Juries  struggle  with  technical  evidence  (e.g.,black  box data),often  favoring  driver  fault  in  comparative  negligence  states.Marketing  claims  invite   fraud   theories,while   supply   chain   involvement(e.g.,sensor   suppliers)complicates   joint

liability.Broader  data  shows  Tesla   reporting  the   most  ADAS  crashes(2019-2024),fueling  scrutiny.Hypothetical  scenarios,like  a  Level  2  system  failing  in  fog  outside  its  ODD,might shift  more  blame  to  manufacturers  if warnings were  inadequate.

4.Systematically    Examining    Responsibility     Boundaries Between  the  Driver  and  the  ADAS

In  shared  human-machine  driving(SAE  Levels  1-3),boundaries  hinge  on  control  allocation, system  limitations,and  human  behavior.

Driver  Responsibilities

Drivers  retain  ultimate  accountability,expected  to   remain  vigilant,override  systems  when necessary,and  adhere  to  ODDs.Negligence  arises  from  distraction   (e.g.,phone  use)or    disabling  features.However,if  ADAS  induces  complacency  via  poor  HMI,courts  may

apportion   fault   proportionally.

ADAS/System Responsibilities

The system  bears  responsibility for defects,such  as sensor failures or algorithmic errors  in foreseeable  scenarios.Manufacturers  are  liable  if  updates  introduce  vulnerabilities  or  if

marketing  misrepresents  autonomy.In  shared  control,liability  shifts  toward  the  machine  as autonomy  increases,but  drivers  must  monitor  (e.g.,hands-on  requirements).

Boundary Factors

·Control  Dynamics: If ADAS  handles  dynamic  driving  tasks  (e.g.,steering),fault  leans manufacturer-ward;monitoring   failures    implicate    drivers.

·Foreseeability:Systems   must   manage   predictable   risks;exceeding   ODD   transfers burden to  users.

·Shared   Models:Joint   liability  applies,with  percentages  based  on  evidence(e.g.,33% system  in  Benavides).Vehicle  owners  act  as  "technological  custodians,"liable  for

neglecting    updates.

This  framework   promotes  fairness   but  requires  robust  data(e.g.,event  recorders)for adjudication.

## 5.Clear,Actionable Regulatory Guidelines or

Recommendations

To  address  gaps,regulators  should  implement  the  following:

1   Adont  Tiered  l  iahilitv  Reaimes·Fnact  laws  assianina  nrimarv  strict  liahilitv  to

·

.

P             v-       ~…J                     v ·         i      i          u            … ·         ·        J                    ~…     J

manufacturers  for   Level  2+defects,with  rebuttable  presumptions  shifting  based  on  ODD  compliance.Inspired  by  EU  Directive  2024/2853,include  mandatory  insurance for ADAS risks.

2.Mandate   Standardization   and    Calibration: Support  bills  like  the  SEMA-backed

federal   ADAS    legislation(2025)for   calibration    guidelines,especially   post-repair,to reduce  malfunctions.Require  industry-wide  HMI  standards  to   minimize

complacency.

3.Enhance   Data   Transparency: Expand  NHTSA's  crash  reporting  to  include  real-time ADAS  data  sharing  for  investigations,per  the  ADS  Evaluation  Program.Mandate

black  boxes  logging  human  inputs and system decisions.

4.Marketing   and    Education   Oversight: Prohibit    exaggerated   claims(e.g.,"full    self- driving")without  disclaimers;require  driver  training  certifications  for  ADAS   use.

5.International       Harmonization: Align US and EU standards via UNECE to facilitate global  compliance,reducing  burdens  on  manufacturers  while  prioritizing  safety.

These  measures  would  clarify  boundaries,deter  negligence,and  support  ADAS  adoption.

Conclusion

Liability in ADAS accidents demands nuanced integration of technical realities and legal

principles.While  drivers  maintain  core  duties,manufacturers  increasingly  shoulder

responsibility  for  system  flaws  amid  shared  control.Case  law  reveals  ongoing  tensions,but proactive  regulations  can  mitigate  risks.Implementing  the  proposed  guidelines  will  foster   safer  roads,equitable  accountability,and  technological  progress  by  2030.