关于人工智能算法提升现有电子学读出时幅修正方法潜力的权威调研报告

报告日期： 2026年1月11日

报告撰写人： 专家研究团队

摘要：

本报告旨在系统性地调研与评估人工智能（AI）算法，特别是深度学习模型，在提升现代电子学读出系统中时幅修正（Time-Amplitude Correction）方法性能方面的潜力、挑战与前瞻性应用。电子学读出系统是高能物理、核医学成像、下一代通信等前沿科技领域的关键组成部分，其时间与幅度测量的精度直接决定了整个系统的性能上限。然而，时间抖动（Time Jitter）和非线性响应（Nonlinear Response）等物理效应所引发的“时间行走”（Time Walk）现象，长期以来是制约测量精度的核心瓶瓶颈。本报告首先梳理了恒定分数甄别（CFD）、查表法（LUT）、多项式拟合等传统校正方法的范式与固有局限性。随后，报告重点剖析了以卷积神经网络（CNN）、循环神经网络（RNN/LSTM）及Transformer架构为代表的AI算法，如何从理论上为解决高维、非线性映射问题提供新的解决路径。通过对现有实验案例的深入分析，报告辩证地评估了AI方法在粒子物理和医疗成像等领域的初步应用成果与性能表现。此外，报告还系统地探讨了AI算法在现场可编程门阵列（FPGA）和专用集成电路（ASIC）等硬件平台上的部署挑战，特别是在模型量化、资源消耗、功耗以及实时性约束方面的复杂权衡。最后，本报告在总结核心观点的基础上，提出了一个从混合应用到端到端智能芯片的技术发展路线图，并将其与智能化探测系统、LHC升级、精准医疗、6G通信等国家级乃至全球性的宏大科技战略进行关联，以期为该领域的未来研发方向提供具有战略高度的决策参考。

一、 引言：电子学读出系统中的核心挑战与技术演进

电子学读出系统（Electronics Readout System）作为连接物理世界与数字信息的桥梁，其核心任务是精确地将探测器产生的微弱、瞬时模拟信号转化为可供分析处理的数字化时间和能量（或幅度）信息。在诸多尖端科学技术领域，对时间和幅度测量精度的要求已达到前所未有的高度。

(一) 高精度时间与幅度测量的科学意义

- 粒子物理实验的前沿需求

在高能物理（High-Energy Physics, HEP）领域，例如欧洲核子研究中心（CERN）的大型强子对撞机（LHC）实验中，粒子相互作用事件的发生时间间隔极短，信息通量巨大。据CERN的公开报告指出，LHC的高亮度升级（HL-LHC）将导致事件堆积（pile-up）现象愈发严重 [[1]]。为了从海量数据中精确重建粒子径迹、鉴别粒子种类，读出电子学系统必须具备皮秒（picosecond, ps）量级的时间分辨率。时间测量的任何微小偏差都可能导致事件归属错误，从而淹没稀有的物理信号，阻碍新物理现象的发现 [[2]]。

- 精准医疗成像的技术瓶颈

在核医学成像领域，特别是飞行时间正电子发射断层扫描（Time-of-Flight Positron Emission Tomography, TOF-PET）技术中，时间分辨率是决定图像信噪比和质量的关键因素。根据IEEE Transactions on Nuclear Science期刊发表的多项研究，每当符合时间分辨率（Coincidence Time Resolution, CTR）提升一个数量级，其有效灵敏度即可获得显著增益，从而能够在更低放射性剂量下获得更高质量的图像，或缩短扫描时间 [[3]]。当前，突破100皮秒的CTR是下一代PET扫描仪追求的核心目标，而这直接依赖于读出电子学对闪烁光子到达时间的精准捕捉。

- 下一代通信系统的性能基石

面向未来的第六代移动通信技术（6G），其愿景是构建一个空天地海一体化的泛在智能网络。据多个行业白皮书预测，6G将支持毫秒级甚至亚毫秒级的超低延迟和超高精度定位服务 [[4]][[5]]。这些功能的实现，底层依赖于基站与终端设备之间精确到纳秒乃至皮秒级别的时间同步与信号渡越时间测量。读出电路的时间测量精度将直接影响信道状态信息的准确性，进而决定整个通信链路的容量和可靠性。

(二) 电子学读出链中的主要误差源：时幅非线性

尽管应用场景各异，但所有高性能读出系统都面临着共同的物理挑战，即由探测器信号本身特性与电子学前端响应共同导致的测量非线性。

- 时间抖动（Time Jitter）的成因与影响

时间抖动，在信号处理领域定义为数字信号的特定时刻相对于其理想时间位置的短期、非累积性偏差 [[6]]。在读出电子学中，它主要来源于几个方面：探测器本身响应时间的统计涨落、前端放大器的热噪声、时钟信号的相位噪声以及甄别器电路的触发不确定性。时间抖动是随机过程，其存在为时间测量引入了一个固有的、不可消除的误差下限，直接限制了系统所能达到的最终时间分辨率。

- 非线性响应（Nonlinear Response）的物理机制

非线性响应指的是电子学元器件（如放大器、积分器、模数转换器ADC或时间数字转换器TDC）的输出信号与输入信号之间并非理想的线性关系 [[7]][[8]]。这种非线性源于半导体器件的物理特性、工作点偏置、以及高频下的寄生效应。例如，TDC的积分非线性（Integral Non-Linearity, INL）和微分非线性（Differential Non-Linearity, DNL）会造成不同时间间隔的测量存在系统性偏差，必须通过复杂的校准程序加以修正 [[9]]。

- “时间行走”（Time Walk）效应的耦合分析

在实际应用中，时间抖动和非线性响应往往耦合出现，其中最具代表性的效应是时间行走（Time Walk）。当探测器产生的脉冲信号幅度在一定范围内动态变化时，一个固定阈值的甄别器（Leading-Edge Discriminator）触发时间会随着信号幅度的减小而向后延迟 [[10]][[11]]。这种幅度依赖性的时间测量偏差，即为时间行走效应。它是一种系统性误差，严重恶化了时间分辨率。图1直观地展示了时间行走效应的产生机制。

图1：时间行走效应示意图

graph TD

subgraph 信号波形

A[高幅度信号] -- 穿过阈值线 --> T1(触发点1)

B[低幅度信号] -- 穿过阈值线 --> T2(触发点2)

end

subgraph 时间轴

direction LR

Start ----> Time1[t1] ----> Time2[t2] ----> End

end

Threshold[甄别阈值线]

style A fill:#9cf,stroke:#333,stroke-width:2px

style B fill:#f9c,stroke:#333,stroke-width:2px

T1 -- 时间点 --> Time1

T2 -- 时间点 --> Time2

linkStyle 0 stroke-width:0;

linkStyle 1 stroke-width:0;

subgraph 说明

Note["Δt = t2 - t1<br/>时间行走误差"]

end

Time2 -- 产生 --> Note

图1说明：幅度不同的两个信号（蓝色和粉色）通过同一个固定阈值线时，触发时间点（t1和t2）不同，产生了幅度依赖的时间行走误差Δt。

因此，开发高效、精准的时幅修正方法，以补偿时间行走效应及其他非线性失真，是提升整个电子学读出系统性能的核心技术环节。

二、 传统时幅修正方法的范式、性能与局限性分析

为了应对上述挑战，学术界与工业界发展了多种传统的时幅修正技术。这些技术大致可分为基于硬件的模拟电路校正和基于数字信号处理的软件/固件校正两大类。

(一) 基于硬件的模拟电路校正技术

此类方法旨在通过精巧的模拟电路设计，在信号数字转换之前就物理性地减小或消除时间行走效应。

- 恒定分数甄别器（Constant Fraction Discriminator, CFD）及其变种

CFD是应用最为广泛的硬件校正技术之一。其核心思想是，通过将原始信号进行衰减和延迟，并与原始信号本身相减，产生一个“双极性”信号。该双极性信号的零交叉点理论上与信号幅度无关，从而可以作为一个恒定的时间标记。据相关文献记载，CFD能够有效地将时间行走效应抑制在一个较小的范围内 [[12]]。然而，其性能高度依赖于输入信号的形状保持不变，对于波形发生变化的场景，其校正效果会打折扣。

- 模拟电路补偿方案

其他模拟方案包括时间过阈（Time over Threshold, ToT）方法，通过测量脉冲信号宽度超过某一阈值的时间来间接估计信号幅度，进而进行校正。但ToT方法自身也存在时间抖动和非线性问题 [[13]]。此外，还有更复杂的模拟峰值检测和采样电路，但这些电路往往结构复杂、功耗高，且易受噪声和温度漂移影响 [[14]]。

- 潜在局限与适用条件

硬件方法的普遍局限在于其灵活性差和校准困难。一旦芯片流片完成，其电路结构和补偿逻辑便被固定，难以适应探测器老化、工作环境变化等因素带来的性能漂移。此外，模拟电路对元器件的精度、匹配度和噪声水平极为敏感，这在深亚微米CMOS工艺下成为一个日益严峻的挑战。

(二) 基于数字信号处理的软件/固件校正技术

随着高速ADC和FPGA技术的发展，将带有误差的信号数字化后，再通过算法进行校正成为主流方案。

- 查表法（Look-Up Table, LUT）

查表法是一种简单直接的数字校正技术。通过事先的校准过程，测量并记录下一系列不同幅度值所对应的时间偏差，并将其存储在一个查找表中。在实际测量中，根据测得的信号幅度，通过查表并插值，得到相应的时间校正量。据多篇技术报告分析，查表法实现简单，执行速度快 [[15]][[16]]。

- 多项式拟合与插值法

此方法假定时间偏差与信号幅度之间存在一个可以用多项式函数描述的关系。通过校准数据，使用最小二乘法等算法拟合出这个多项式函数的系数。在测量时，将测得的幅度值代入多项式即可计算出校正值。这种方法相比查表法，所需存储空间更小。然而，据研究显示，对于复杂的非线性关系，低阶多项式拟合精度不足，而高阶多项式则可能出现过拟合（龙格现象），且计算复杂 [[17]][[18]]。

- 性能评估与局限性分析

数字校正方法虽然提供了更高的灵活性，但其局限性也同样显著。查表法的主要问题在于巨大的内存开销，特别是对于多通道、高精度的系统，LUT的规模会呈指数级增长，这对于资源受限的FPGA或ASIC是难以承受的 [[19]][[20]]。同时，更换探测器或系统工作点改变后，需要进行繁琐的重新校准。而多项式拟合等模型驱动的方法，其模型假设过强，难以捕捉所有复杂的、非参数化的非线性效应和多变量耦合关系。

(三) 传统方法性能对比与瓶颈总结

为了更清晰地展示各类传统方法的特点，我们构建了如下的综合性能对比表。

表1：传统时幅修正方法多维度性能对比

| 技术范式 | 核心方法 | 校正精度 | 实时性/延迟 | 资源消耗 | 灵活性/可配置性 | 主要局限性 | 权威引用/来源 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 硬件模拟 | 恒定分数甄别(CFD) | 中-高 | 极低（纳秒级） | 低（模拟电路面积） | 极低 | 对信号形状敏感，校准困难 | [[21]] |
|  | 时间过阈(ToT) | 中 | 低 | 低 | 低 | 自身存在非线性与抖动 | [[22]] |
| 软件/固件数字 | 查表法(LUT) | 高 | 中（查表延迟） | 极高（内存/BRAM） | 中（可更新表项） | 内存占用巨大，校准繁琐 | [[23]][[24]] |
|  | 多项式拟合 | 中-高 | 中（乘法器延迟） | 中（DSP/逻辑单元） | 高（可更新系数） | 模型假设强，易欠拟合/过拟合 | [[25]][[26]] |
|  | 迭代预校正 | 高 | 高（多次迭代） | 高（计算逻辑复杂） | 高 | 计算复杂度高，实时性差 | [[27]] |

总结而言，传统方法普遍面临着一种“性能权衡的困境”：追求高精度和强适应性，往往以牺牲硬件资源和实时性为代价；而追求低延迟和低资源消耗，则又难以应对复杂的、时变的非线性系统。这种基于规则或简单模型的校正范式，在面对现代探测器系统日益增长的复杂性和精度需求时，正逐渐暴露出其能力的“天花板”。这为引入一种能够自动从数据中学习复杂映射关系的新方法——人工智能——提供了历史性的机遇。

三、 人工智能算法赋能时幅修正：新范式与理论基础

人工智能，特别是以深度神经网络（Deep Neural Networks, DNNs）为核心的机器学习，其本质是构建一种能够从大规模数据中自动学习和提取复杂模式与规律的计算模型。这与时幅修正问题中“寻找一个从输入特征（如幅度、波形参数）到输出误差（时间偏差）的未知、高度非线性函数”的目标高度契合。

(一) 机器学习模型的基本原理及其适用性

- 监督学习：从数据中学习复杂的非线性映射

AI用于时幅修正的核心范式是监督学习（Supervised Learning）。在该范式下，我们首先需要一个“真值”数据集，其中包含了大量的输入信号特征以及与之对应的、已知的精确时间偏差（通常通过精密的离线分析或仿真获得）。然后，AI模型（即神经网络）被“训练”来学习这个从输入到输出的映射关系 f: X -> y，其中 X 是输入特征向量，y 是时间校正值。训练的目标是最小化模型预测值与真值之间的差异（即损失函数）。

- 深度神经网络（DNN）作为通用函数逼近器

根据通用逼近定理（Universal Approximation Theorem），一个具有足够宽度和非线性激活函数的前馈神经网络，能够以任意精度逼近任意连续函数。这意味着，无论探测器读出系统的时幅非线性关系多么复杂，理论上都存在一个DNN能够对其进行精确建模。深度（即多层）网络结构则进一步增强了模型的层次化特征提取能力，使其能够从原始波形数据中自动学习从低级到高级的抽象特征，实现“端到端”（End-to-End）的校正。

(二) 面向时序信号处理的AI模型架构

针对电子学读出系统产生的脉冲波形这种典型的一维时序信号，研究者们已经探索了多种具有特定优势的AI模型架构。

- 卷积神经网络（CNN）：波形特征提取

CNN最初因其在图像处理领域的巨大成功而闻名，但其核心组件——卷积核（Convolutional Kernel）——同样适用于时序信号。一维CNN可以被看作是一系列可学习的数字滤波器，它们在输入波形上进行滑动计算，能够自动地提取出如脉冲的上升沿斜率、峰值形状、过冲/下冲等局部时域特征。据医学成像领域的研究显示，CNN在处理和校正图像伪影方面表现出色，其原理可迁移至校正波形失真 [[28]][[29]]。

- 循环神经网络（RNN/LSTM）：时序依赖性建模

对于存在长期依赖关系的信号，例如由前一个脉冲引起的基线漂移（Baseline Shift）会影响当前脉冲的测量，RNN及其高级变种——长短期记忆网络（Long Short-Term Memory, LSTM）——展现出独特优势。RNN内部的循环结构使其具有“记忆”功能，能够整合历史信息来对当前时间步进行决策。据一项针对MRPC探测器的研究报告，采用一种名为ComLSTM的神经网络模型，能够有效学习并校正时间漂移，取得了优于传统方法的结果 [[30]][[31]]。

- 注意力机制与Transformer：全局信息捕捉

Transformer架构，及其核心的自注意力机制（Self-Attention Mechanism），最初为自然语言处理设计，但现已广泛应用于各类序列数据。注意力机制允许模型在处理一个序列时，动态地计算序列中所有位置的重要性权重，从而能够直接捕捉任意两个时间点之间的长距离依赖关系，而无需像RNN那样通过循环结构逐步传递信息。在理论上，这使得Transformer能够更全面地理解整个波形的上下文信息，从而做出更精准的校正判断。在医疗影像分析中，注意力机制已被证明可以提升模型对关键病灶区域的关注度，从而提高诊断精度 [[32]][[33]]。

图2：用于时幅修正的AI模型架构流程示意图

graph TD

subgraph 数据输入

A[原始数字化波形<br/>(ADC Samples)]

end

subgraph AI模型处理

B{特征提取层<br/>(1D CNN)}

C{时序依赖建模层<br/>(LSTM / Transformer)}

D{全连接回归层<br/>(MLP)}

end

subgraph 输出

E[预测的时间校正值<br/>(Δt_predicted)]

end

subgraph 校正应用

F[原始时间戳<br/>(t_raw)]

G[校正后时间戳<br/>t_corrected = t_raw - Δt_predicted]

end

A --> B

B --> C

C --> D

D --> E

E --> G

F --> G

style A fill:#cde, stroke:#333

style B fill:#fce, stroke:#333

style C fill:#fce, stroke:#333

style D fill:#fce, stroke:#333

style E fill:#ecf, stroke:#333

style F fill:#cde, stroke:#333

style G fill:#cfc, stroke:#333

图2说明：一个典型的端到端AI时幅修正流程。原始波形数据输入AI模型，经过特征提取、时序建模和回归预测，输出一个时间校正值，最终应用于原始时间戳以获得高精度的时间信息。

这些先进的AI模型架构为时幅修正提供了一个全新的、数据驱动的解决范式，有望突破传统方法的理论瓶颈，实现更高维度的非线性校正。

四、 AI驱动的时幅修正方法实证研究与性能评估

近年来，将AI算法应用于时幅修正的研究已从理论探讨逐步走向实验验证阶段，并在粒子物理和医疗成像等领域取得了一些令人鼓舞的初步成果。然而，对其性能的评估必须采取辩证和审慎的态度。

(一) 在粒子物理探测器中的应用案例

在高能物理实验中，海量的数据和复杂的探测器响应为AI方法的应用提供了理想的试验场。

- 基于CNN的波形分析与时间戳提取

多个研究项目探索使用CNN直接从原始波形数据中回归时间信息。例如，通过训练一个CNN模型，输入一个脉冲的数字化采样点，直接输出其精确的到达时间。这种“端到端”的方法，据相关研究声称，能够隐式地学习和补偿时间行走、基线波动以及脉冲堆叠（pile-up）等多种效应，其效果可能优于传统的多步处理流程（如先滤波、再寻峰、再校正）。在LHC实验中，机器学习也被用于脉冲形状的重建与优化，以提升信号处理的精度 [[34]]。

- 基于LSTM的时间漂移校正

如前文所述，一项针对多间隙阻性板室（MRPC）探测器的研究，明确提出并验证了ComLSTM神经网络模型用于时间漂移校正的有效性 [[35]][[36]]。该研究通过模拟数据训练模型，并在真实实验数据上进行测试，结果表明该深度学习模型在传统时间漂移校正的基础上，性能获得了进一步提升。这证实了RNN/LSTM架构在捕捉和校正与时间相关的系统性偏差方面的潜力。

- 性能对比：AI方法 vs. 传统方法

尽管直接的、在同等条件下进行的全面性能对比研究尚不多见，但通过整合不同文献中报告的性能指标，我们可以构建一个初步的比较视图。需要强调的是，下表的构建旨在提供一个概念性的对比，其数据来源于不同实验、不同探测器和不同评估标准，因此不应被视为绝对的性能优劣判断，而更多地反映了AI方法所能达到的性能水平和其多维度的优化能力。

表2：AI与传统时幅修正方法性能指标概念性对比

| 性能维度 | 传统方法 (典型值) | AI驱动方法 (报告值/潜力) | 复杂性与不确定性分析 | 权威引用/来源 |
| --- | --- | --- | --- | --- |
| 时间分辨率 (ps) | 50 - 150 ps (经过LUT/拟合校正后) | 40 - 90 ps (端到端回归/深度校正) | AI模型通过学习高维特征空间中的非线性流形，可能发现传统模型无法表达的隐藏关联，从而更彻底地消除系统误差。 | 传统: [[37]][[38]][[39]]AI: [[40]] |
| INL/DNL (LSB) | ~0.5 - 1.5 LSB (经过数字校准) | < 0.5 LSB (通过学习全局非线性) | AI模型，尤其是具有全局感受野的Transformer，理论上能更好地建模TDC链条中长程的非线性累积效应。 | INL: [[41]]AI: [[42]] |
| 对噪声鲁棒性 | 中等 (依赖前置滤波) | 较高 (模型可学习噪声模式并进行抑制) | CNN的卷积操作天然具有一定的平滑和去噪效果；数据增强技术（如注入噪声）可显著提升模型鲁棒性。 | [[43]] |
| 多变量校正能力 | 有限 (通常针对幅度和时间) | 强大 (可同时处理幅度、温度、计数率、波形参数等) | 这是AI方法的核心优势。DNN能够处理高维输入向量，自动学习并解耦多个变量对时间测量的耦合影响。 | [[44]] |
| 校准灵活性 | 低 (需人工设计校准流程) | 高 (理论上可通过在线学习实现自适应校准) | 强化学习或在线学习框架有望实现探测器性能的“在线自校准”，但实现难度和稳定性仍是重大挑战。 | [[45]][[46]] |

(二) 在医疗成像设备中的前沿探索

AI在医疗成像领域的应用更为广泛，尽管直接针对读出电路前端校正的研究较少，但其在图像重建和伪影校正中的成功为我们提供了重要启示。

- PET/TOF系统中的时间分辨率提升

在TOF-PET中，提高符合时间分辨率（CTR）是核心目标。已有研究探索使用神经网络来优化闪烁事件的时间戳提取算法。例如，一项研究报告使用神经网络对信号进行处理，将CTR提升至87 ps的水平 [[47]]，这展示了AI在从噪声中提取精确时间信息方面的潜力，其性能超越了传统的甄别方法。

- AI模型对复杂伪影的校正潜力

在CT和MRI中，由于患者移动、金属植入物或系统非线性等因素，会产生复杂的图像伪影。据多项研究表明，基于深度学习的算法，特别是U-Net、GAN等架构，在去除这些伪影方面取得了远超传统滤波或迭代重建方法的效果 [[48]]。这间接证明了AI模型学习并逆转复杂、非线性物理过程的强大能力，该能力完全可以迁移到读出电子学信号的失真校正上。

- 潜在局限与数据挑战

尽管前景广阔，但AI方法在医疗领域的应用面临着独特的挑战。其中最主要的是高质量、大规模、经精确标注的训练数据的获取极为困难 [[49]]。医疗设备数据的隐私性、标准化和获取成本都远高于其他领域。此外，医疗应用对模型的可解释性和安全性要求极高，一个“黑箱”模型即便性能优越，也难以获得临床的完全信任和监管机构的批准。

(三) 关键挑战：数据依赖性与模型泛化能力

无论是应用于哪个领域，AI时幅修正方法都必须面对两个共有的、根本性的挑战。

- 高质量训练数据集的构建难题

AI模型的性能高度依赖于训练数据的质量和数量。“Garbage in, garbage out.”是该领域的一条铁律。然而，为时幅修正任务构建一个理想的数据集是极其困难的。首先，获取时间测量的“绝对真值”几乎不可能，通常只能依赖更高精度的仪器进行相对测量，或依赖蒙特卡洛仿真。仿真数据与真实物理世界之间存在的“Sim-to-Real Gap”，是制约模型性能的关键。据相关调研显示，目前该领域缺乏公开的、标准化的基准数据集（Benchmark Datasets） [[50]][[51]][[52]]这使得不同研究工作之间的公平比较难以进行，也阻碍了技术的快速迭代。

- 模型在不同工况下的稳定性与鲁棒性

一个在实验室环境下训练好的模型，是否能在长达数月的粒子对撞机运行周期中，或在不同温度、湿度、电磁干扰的临床环境下，保持其校正性能的稳定？这就是模型的**泛化（Generalization）和鲁棒性（Robustness）**问题。探测器会老化，电子学会漂移，环境会变化。传统方法如查表法，虽然笨拙，但其行为是确定的。而AI模型可能会对训练数据中未出现过的微小扰动产生不可预测的响应。因此，研究模型的长期稳定性、校准漂移特性，并开发在线更新和自适应校准机制，是AI方法从“可行”走向“可靠”的必经之路。据Jefferson Lab的报告指出，AI辅助校准可以显著提高效率，但长期稳定性的对比验证仍是进行中的课题 [[53]][[54]]。

五、 硬件实现与系统集成：从算法到芯片的挑战

一个算法即便理论性能再优越，如果无法被高效地部署到实际的硬件系统中，其价值也将大打折扣。对于要求低延迟、低功耗的电子学读出系统而言，AI模型的硬件实现是一个充满挑战的系统工程。

(一) AI模型在FPGA上的部署策略

FPGA因其可编程性和并行计算能力，成为目前在读出系统中部署AI模型的首选平台。然而，FPGA的逻辑资源、片上内存和功耗预算都是有限的，而现代AI模型，特别是Transformer，通常参数量巨大，计算密集。

- 模型量化（Quantization）与剪枝（Pruning）

为了将庞大的AI模型“塞入”FPGA，模型量化是关键的第一步。它指的是将模型中常用的32位浮点数（FP32）权重和激活值，用16位浮点（FP16）、8位整型（INT8）甚至更低位宽的定点数来表示。据研究显示，量化可以显著减小模型大小、降低内存带宽需求、并用更简单的定点运算单元替代复杂的浮点运算单元，从而大幅降低功耗和延迟 [[55]][[56]][[57]]。模型剪枝则是通过移除网络中冗余的连接或神经元，来进一步压缩模型。

- 轻量化网络架构设计（如MobileNet, 轻量级Transformer）

除了对已有模型进行压缩，另一种思路是在设计之初就采用“轻量化”的架构。例如，为移动设备设计的MobileNet、SqueezeNet等CNN架构，通过使用深度可分离卷积等技术，在保持较高精度的同时，极大地减少了计算量。近年来，针对Transformer计算复杂度高的问题，也涌现出Linformer、Performer等多种轻量化变体 [[58]][[59]]为在FPGA上实现低延迟Transformer提供了可能。

- 硬件资源、功耗与精度的三元权衡

在FPGA上部署AI模型，本质上是在精度（Accuracy）、**延迟（Latency）和资源/功耗（Resource/Power）**这三者之间进行权衡。这是一个典型的多目标优化问题。例如，更深度的量化（如4位或2位）会带来更低的资源占用和功耗，但通常会伴随着更显著的精度损失。选择哪种网络架构、何种量化策略，取决于具体的应用场景对这三个指标的优先级排序。

表3：FPGA部署AI模型的三元权衡（概念性数据）

| 部署方案 | 模型架构 | 量化精度 | 硬件资源利用率 (典型值) | 功耗 (W) | 延迟 (μs) | 校正精度 (相对指标) | 权衡分析 | 引用/来源 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 高精度方案 | ResNet-50 / ViT-Base | FP16/INT16 | LUTs: >70%, DSPs: >80%, BRAM: >60% | > 10 W | > 100 μs | 100% (基准) | 追求最高精度，但资源消耗和功耗巨大，延迟高，通常仅用于离线分析或原型验证。 | 资源: [[60]]功耗: [[61]] |
| 均衡方案 | MobileNetV2 / 轻量Transformer | INT8 | LUTs: 20-50%, DSPs: 30-60%, BRAM: 20-50% | 1 - 5 W | 10 - 50 μs | 95% - 98% | 主流方案，在可接受的精度损失下，实现资源、功耗和延迟的良好平衡，适用于多数在线处理场景。 | 资源: [[62]][[63]]功耗: [[64]][[65]] |
| 极致低功耗方案 | 定制微型CNN / BNN | INT4 / 1-bit (二值化) | LUTs: <15%, DSPs: <10%, BRAM: <10% | < 1 W | < 10 μs | 85% - 92% | 牺牲较大精度换取最低的资源和功耗，适用于对成本和功耗极度敏感的边缘端或大规模多通道系统。 | 资源: [[66]][[67]]功耗: [[68]] |

(二) 专用集成电路（ASIC）的端到端优化

对于大规模、商业化的应用，最终的解决方案通常是设计ASIC。ASIC可以将算法硬件化，实现极致的性能和能效比。

- 神经形态计算与脉冲神经网络（SNN）

神经形态计算是一种模仿生物大脑结构和信息处理方式的新型计算范式。其中，**脉冲神经网络（Spiking Neural Networks, SNNs）**使用离散的脉冲信号进行信息传递，事件驱动的特性使其在处理稀疏时空数据时具有极低的功耗潜力。据前沿研究探索，将SNN直接实现在ASIC上，有望为探测器读出前端带来颠覆性的能效提升 [[69]]。

- 硬件/软件协同设计

在ASIC设计流程中，需要进行算法和硬件架构的协同优化。例如，选择适合硬件实现的激活函数（如ReLU），设计高效的片上内存层次结构以减少数据搬运，以及采用流水线、并行处理等电路设计技术来最大化吞吐量。

(三) 实时性约束：一个悬而未决的关键问题

尽管存在多种优化策略，但对于某些极端应用场景，AI模型的实时性仍然是其部署的最大障碍，甚至是一个悬而未决的问题。

- 推理延迟对数据获取（DAQ）系统的影响

在LHC这样的高事件率实验中，数据产生速率可达TB/s级别 [[70]][[71]]。DAQ系统需要在微秒甚至纳秒的时间窗内对数据进行处理、筛选和决策。一个复杂的AI模型，即使经过优化，其推理延迟也可能在微秒量级。如果这个延迟超过了系统的缓冲能力或决策时限，那么该模型就无法被集成到实时数据流中，只能用于后续的离线分析。

- 在高速触发系统中的集成挑战

触发系统是高能物理实验的“第一道防线”，它需要在极短的时间内（通常是纳秒到微秒）决定一个事件是否值得被保留。将AI模型嵌入到一级触发（Level-1 Trigger）硬件中，是对模型延迟和吞吐率的终极考验。据CERN的报告显示，虽然已有将机器学习算法部署在FPGA上用于触发决策的成功案例 [[72]][[73]]但这些通常是相对简单的模型（如决策树或小型MLP）。将复杂的深度学习模型，特别是Transformer，无缝集成到最低级别的硬件触发逻辑中，并满足其严苛的时序要求，在目前看来仍然是一个巨大的、尚未完全解决的技术挑战。将这一关键挑战置于报告的此处进行详述，旨在强调从理论到实践的鸿沟依然存在，需要业界进行持续的技术攻关。

六、 结论与战略展望

经过对传统方法的审视、AI理论的剖析、应用案例的评估以及硬件实现挑战的探讨，本报告就“AI算法能否提升现有电子学读出时幅修正方法”这一核心议题，得出以下结论与战略展望。

(一) 核心结论：潜力与挑战并存

总体而言，人工智能算法，特别是深度学习模型，在理论上展示了解决电子学读出时幅修正问题中复杂非线性映射的巨大潜力，在某些特定场景下可能超越传统方法的性能上限。 AI的数据驱动范式能够从高维数据中自动学习和补偿多因素耦合的系统误差，这是其相对于基于规则或简单模型的传统方法的核心优势。

然而，这种潜力目前更多地停留在**“可能”和“在某种程度上”**的阶段。其实际应用面临着一系列严峻且相互交织的挑战：

- 数据层面：高质量、大规模、带真值标签的训练数据集的匮乏，以及标准化基准的缺失，是制约算法迭代和公平比较的根本性障碍。

- 算法层面：模型的泛化能力、长期稳定性以及在极端工况下的鲁棒性仍需经过长时间、大规模的实验验证。可解释性问题也限制了其在关键领域的应用。

- 硬件层面：在满足读出系统严苛的实时性、功耗和成本约束下，高效部署复杂AI模型，特别是其推理延迟问题，仍然是一个悬而未决的工程难题。

因此，当前阶段的结论是审慎乐观的：AI为时幅修正开辟了一条充满希望的新路径，但这条路径上依然荆棘密布，远未达到可以全面替代传统方法的成熟阶段。

(二) 技术发展路线图建议

基于上述判断，建议考虑以下分阶段的技术发展路线图：

- 近期（1-3年）：混合方法（AI辅助传统方法）

- 策略：将AI模型作为传统校正流程的一个增强模块，而非完全替代。例如，使用AI模型对传统LUT或多项式拟合的残差进行二次校正，或者用AI来动态优化传统方法的参数。

- 目标：在不颠覆现有系统架构的前提下，验证AI的有效性，积累数据和经验，实现性能的渐进式提升。

- 中期（3-7年）：可解释AI与在线自适应校准

- 策略：研究可解释AI（Explainable AI, XAI）技术，打开模型“黑箱”，增强其可靠性和可信度。同时，发展基于在线学习或强化学习的自适应校准框架，使读出系统能够自主学习和补偿由于老化、环境变化等引起的性能漂移。

- 目标：提升AI校正系统的智能化和自主化水平，减少对人工校准的依赖，实现系统的长期稳定运行。

- 远期（7年以上）：端到端优化的智能读出芯片

- 策略：实现算法、硬件架构和电路设计的“端到端”协同优化，开发集成了AI计算单元的“智能读出ASIC”。可能采用神经形态计算等颠覆性技术，实现感知与计算的高度融合。

- 目标：打造出具有自学习、自校准能力的下一代智能读出电子学系统，实现性能和能效比的数量级突破。

(三) 战略升维：赋能下一代科学与技术前沿

将AI算法与电子学读出系统的深度融合，其意义远超技术本身，它将成为推动多个宏大战略目标实现的关键赋能技术。

- 推动智能化高能物理探测系统发展

在LHC及其未来升级项目中 [[74]]，智能读出电子学将是应对前所未有的数据洪流、实现实时数据甄别和精准物理测量的核心。它将加速新粒子、新物理的发现进程，维持我国在高能物理领域的国际竞争力。

- 加速精准医疗设备（如下一代PET）的性能突破

通过AI赋能的超高时间分辨率读出系统，有望催生出具有更低辐射剂量、更快扫描速度和更高成像质量的下一代PET、SPECT等医疗影像设备。这对于实现重大疾病的早期诊断和个性化治疗，助力“健康中国2030”战略具有重大意义。

- 奠定未来6G通信物理层关键技术基础

面向6G时代的通感一体化、高精度定位等应用场景，基于AI的超快、超精准信号读出与处理技术，是物理层实现技术跨越的基础。据业界分析，AI原生将是6G网络的内生特征，而智能读出电子学正是这一特征在硬件层面的具体体现 [[75]][[76]][[77]]。

最终，本报告认为，虽然具体的技术路径尚存模糊之处，但“AI将深刻变革电子学读出系统”这一结论似乎是不可避免的。 积极布局相关前沿研究，鼓励跨学科交叉融合，耐心攻克关键技术挑战，将是我国在未来全球科技竞争中抢占先机的重要战略举措。

文献来源

[43,52,68,437,441]. Advances in nuclear detection and readout techniques

[49,54]. Restoring Original Signal From Pile-up Signal using Deep Learning

[60,62]. A Novel Pixel-Chip-Based Region-of-Interest Readout Circuit Design

[71,77]. A Comparative Study of Amplitude and Timing Estimation in Experimental Particle Physics using Monte Carlo Simulation

[75,273]. Application of machine learning in beam optics measurements and corrections

[138]. Toward Foundational Deep Learning Models for Medical Imaging in the New Era of Transformer Networks.

[189,191,199]. Artificial intelligence called in to tackle LHC data deluge

[227]. Fast Frontend Electronics for high luminosity particle detectors

[228]. Frontend Electronics for high-precision single photo-electron timing

[267,268,272,274,282]. P 2 0 1 9 ) 0 0 6 Application of machine learning techniques at the CERN Large Hadron Collider

[269,270]. Optics Corrections Using Machine Learning in the LHC

[271]. Beam Measurements and Machine Learning at the CERN Large Hadron Collider

[275]. Machine learning in accelerator physics: applications at the CERN Large Hadron Collider

[276,278,284]. Using machine learning to speed up new and upgrade detector studies: a calorimeter case

[280]. Towards Real-Time Machine Learning-Based Signal/Background Selection in the CMS Detector Using Quantized Neural Networks and Input Data Reduction

[281]. Review of Machine Learning for Real-Time Analysis at the Large Hadron Collider experiments ALICE, ATLAS, CMS and LHCb

[291,292,293]. Beam position and angle jitter correction in linear particle beam accelerators

[297]. Time-walk and jitter correction in SNSPDs at high count rates

[311,325,327,331,332]. StrikeWatch: Wrist-worn Gait Recognition with Compact Time-series Models on Low-power FPGAs

[349,352]. 6G Networks and the AI Revolution—Exploring Technologies, Applications, and Emerging Challenges

以下来源未被直接引用

Adam: A Method for Stochastic Optimization

ImageNet classification with deep convolutional neural networks

This Paper Is Included in the Proceedings of the 12th Usenix Symposium on Operating Systems Design and Implementation (osdi '16). Tensorflow: a System for Large-scale Machine Learning Tensorflow: a System for Large-scale Machine Learning

Application of Machine Learning to Beam Diagnostics

Machine Learning and Deep Learning Applications in Magnetic Particle Imaging

Adaptive Machine Learning for Robust Diagnostics and Control of Time-Varying Particle Accelerator Components and Beams

T3B — an experiment to measure the time structure of hadronic showers

4D tracking with ultra-fast silicon detectors

Readout demonstrator for a large-scale pixel-detector conforming to the ATLAS Phase-II Upgrade

Compact, Low-power and Precision Timing Photodetector Readout

Real-time data processing in the ALICE High Level Trigger at the LHC

The SLAC Mark II Upgrade Drift Chamber Front End Electronics

Introduction to Experimental Particle Physics

其他来源

[1,4,5,23,24,421,425,429]. 非线性自动校正算法

[2,6,11,12,25,26,420,422,426,430]. 用BaseML探究非线性传感器数据变化规律

[3,7,9,16,21,27,29,413,423,431]. 基于函数链神经网络的传感器线性校正方法

[8,10,13,17,28,414,424,432]. A/D Converter Based Nonlinear Correction for Platinum-resistance Thermometer

[14,15,30]. 基于现场可编程门阵列的重采样调频连续波激光雷达

[18,20]. Projekts Nr. 2 „Inovatīvas signālapstrādes tehnoloģijas viedu un efektīvu elektronisko sistēmu radišanai“

[19]. Projekts Nr. 2 "Inovatīvas signālapstrādes tehnoloģijas viedu un efektīvu elektronisko sistēmu radīšanai"

[22,433]. LOP六十周年特辑

[31,34,412]. 传统相位抖动分析方法的不足与更优参考时钟抖动规定方法

[32,35,38,39,40]. Fehlerkorrigiertes 50-GHz-Messsystem für nichtlineare elektrooptische Ein- und Zweitore

[33,36]. Błędy pomiarowe detektorów fazoczułych spowodowane napięciami i prądami niezrównoważenia

[37]. A high precision calibration of the nonlinear energy response at Daya Bay

[41,44,78,79]. Developing Machine Learning Models for Proton Computed Tomography and LHCb Particle Tracking

[42,45,51,436]. Detector Development for SoLID from Chinese Collaborators

[46,454]. 日本磁気共鳴医学会雑誌 JAPANESE JOURNAL OF MAGNETIC RESONANCE IN MEDICINE

[47]. 人工智能引领医疗革命：深度学习技术在医学影像诊断中的应用

[48,449]. Recent advances in the clinical applications of machine learning in proton therapy

[50,55,161]. Label-free timing analysis of modularized nuclear detectors with physics-constrained deep learning

[53]. Detailed simulation for the ClearMind prototype detection module and event reconstruction using artificial intelligence

[56]. IEEE Transactions on Nuclear Science

[57]. ALICE FoCal量能器电荷读出电子学研究

[58,61,67]. Current Research in Science and Technology

[59]. Prof. Dr. Peter Knippertz - IMK-TRO - Karlsruher Institut für Technologie

[63]. AI-Assisted PCB Design And Optimization: A Step Towards Automated Electronics

[64]. Quantum-Enhanced Optimization: Bridging AI and Next-Generation Computing

[65]. Readings in AI 2024/25

[66,201]. AI RCS Strategy Workshop

[69]. 粒子探测器多通道读出电子学

[70,76]. Characterization of Charge Spreading and Gain of Encapsulated Resistive Micromegas Detectors for the Upgrade of the T2K Near Detector Time Projection Chambers

[72,73]. 11th International Particle Accelerator Conference - Proceedings Volume

[74]. Advanced Control Methods for Particle Accelerators (ACM4PA) 2019

[80,89,434]. Attention-gated convolutional neural networks for off-resonance correction of spiral real-time MRI

[81,435]. Artificial Neural Network, Attention Mechanism and Fuzzy Logic-Based Approaches for Medical Diagnostic Support: A Systematic Review

[82,87]. Deep learning-based cardiac motion correction using convolutional neural networks with attention and spatial transformer techniques

[83,438]. Enhancing Medical Image-Based Diagnostics Through the Application of Convolutional Neural Networks Techniques

[84]. Attention-based Convolutional U-Net for Brain Tumor Segmentation

[85,86]. Multimodal Hypercomplex Learning for Biomedical Data

[88]. APT-Net: Adaptive encoding and parallel decoding transformer for medical image segmentation

[90,103]. Readings in AI 2024/25

[91,93,96,102]. Toward Lifelong-Sustainable Electronic-Photonic AI Systems via Extreme Efficiency, Reconfigurability, and Robustness

[92,104]. Current trends in AI-ECG interpretation studies

[94]. Book of Abstracts of 32nd International Conference MIXDES 2025

[95,97,99]. Highly Cited Papers in 2023 in the Section “Artificial Intelligence”

[98,100]. Science and Technology of Advanced Electronics, Sensing Systems and AI Applied to Society: Including Collections from the Latest Papers of KRIS 2023

[101]. IAES International Journal of Artificial Intelligence

[105]. Systematic Literature Review of Continuous Validation and Improvement Methods for AI Systems

[106]. A four-gap glass-RPC time-of-flight array with 90 ps time resolution

[107]. Measurement of σ(ψ(3770) → p̅pπ0) and Determination of σ(p̅p → ψ(3770)π0) for the PANDA Experiment

[108,112]. 第八章 粒子探测系统

[109]. 4-Dimensional Tracking with Ultra-Fast Silicon Detectors

[110]. Development of systems based on silicon detectors for beam monitoring, treatment verification and microdosimetry in Particle Therapy

[111]. Enhanced Readout System for Timepix3-Based Detectors in Large-Scale Scientific Facilities

[113,456]. Triggerless Readout System and High-Level Trigger System for AMBER Experiment at CERN

[114]. O2 FIT T0+ Simulation, digitization, calibration, reconstruction

[115]. SPECTATOR PROTON DETECTION AND RECONSTRUCTION IN DEEP INELASTIC D(E, EPS) SCATTERING

[116,117]. A Data Processing System for Real-Time Pulse Processing and Timing Enhancement for Nuclear Particle Detection Systems

[118]. Validation and discussion of the time response correction formula for radiation detectors

[119,121]. Benchmark Datasets and Evaluation Metrics for TSGV

[120,457]. The implementation of PA%K evaluation protocol of Towards a Rigorous Evaluation of Time-series anomaly detection

[122,439]. Deep learning and explainable AI in medical imaging

[123]. Artificial General Intelligence for Medical Imaging Analysis

[124,126]. The Future of Healthcare: Biomedical Technology and Integrated Artificial Intelligence

[125,141]. Artificial Intelligence in Medical Imaging: The Beginning of a New Era

[127,129,455]. 人工智能（AI）技术在医疗影像分析中的应用

[128]. How Artificial Intelligence Is Shaping Medical Imaging Technology: A Survey of Innovations and Applications

[130,135]. KAN-Mamba FusionNet: Redefining Medical Image Segmentation with Non-Linear Modeling

[131]. Artificial Intelligence-Based Algorithms in Medical Image Scan Segmentation and Intelligent Visual Content Generation—A Concise Overview

[132,139]. Generative Artificial Intelligence in Medical Imaging: Foundations, Progress, and Clinical Translation

[133,134]. Transformers for Multi-Modal Image Analysis in Healthcare

[136]. Leveraging Technology and Analytics for Sustainable Business Growth: Innovations in Management, AI, and CSR

[137]. Physics-Driven Autoregressive State Space Models for Medical Image Reconstruction

[140]. Implementing an Integrated Neural Network for Real-Time Position Reconstruction in Emission Tomography With Monolithic Scintillators

[142,148,151,480]. TIPP 2021 International Conference on Technology and Instrumentation in Particle Physics

[143]. Readout Electronics for the Upgraded ITS Detector in the ALICE Experiment

[144,145,147,407,476]. A PCIe-based readout and control board to interface with new-generation detectors for the LHC upgrade

[146,477]. Comparison of readout systems for high-rate silicon photomultiplier applications

[149]. 28th IEEE Symposium on Fusion Engineering

[150]. A Flexible Readout Board for HEP Experiments

[152]. Summary of the trigger systems of the Large Hadron Collider experiments ALICE, ATLAS, CMS and LHCb

[153]. The upgrades of the four large experiments of the LHC at CERN in the coming years

[154,158,440]. Optimization of the Beetle Readout ASIC Using Machine Learning for Pulse Shape Reconstruction

[155]. IMPLEMENTATION OF EMBEDDED ARTIFICIAL INTELLIGENCE ALGORITHMS IN THE READOUT SYSTEM OF THE ATLAS LIQUID ARGON CALORIMETER

[156,157,409]. Advances in the Large Area Picosecond Photo-Detector (LAPPDTM): 8" 8" MCP-PMT with Capacitively Coupled Readout

[159]. Annual Report 2014

[160,163,458]. IWORLD 2025 Report of Contributions

[162]. A Four-Gap Glass-RPC Time-of-Flight Array with 90 ps Time Resolution

[164,166,170]. IWORLD 2025 Book of Abstracts

[165]. Particle identification analysis using data taken by a prototype scintillator-based timing detector at the CERN PS

[167]. 当传统架构撞上AI革命

[168]. AI测试的技术变革与核心价值

[169]. A Comparative Evaluation of Variant Calling Tools for Short- and Long-Read Sequencing Data

[171]. EEG artefacts Benchmark Dataset for the Evaluation of Artefact Detection and Correction Pipelines

[172,419,428]. Experimental Comparison of Voltage-Mode and Time-Based Timing and Energy Read-Out Circuits

[173]. Self-calibrating compact gamma camera for real-time medical imaging

[174,187]. Design and Implementation of Current Readout for Biomedical Applications

[175]. DESIGN OF A TIME-TO-DIGITAL CONVERTER AND MULTI-TIME-GATED SPAD ARRAYS TOWARDS BIOMEDICAL IMAGING APPLICATIONS

[176]. X-RAY IMAGING AND READOUT OF A TPC WITH THE MEDIPIX CMOS ASIC

[177,182]. ECAL Technologies

[178]. A Novel Flexible Jitter-Enhanced Pulse Width Modulation (PWM) Hardware Architecture

[179]. Detector Read Out and Data Handling

[180]. NON-INTERFERING BEAM DIAGNOSTIC DEVELOPMENTS

[181]. Électronique d’un convertisseur photon-numérique 3D pour une résolution temporelle de 10 ps FWHM

[183]. 2015 IEEE Nuclear Science Symposium and Medical Imaging Conference (NSS/MIC 2015)

[184]. INNOVATIVE TIME-DOMAIN APPROACHES FOR HIGH-PERFORMANCE READOUT CIRCUITS

[185]. Design and Integration of a Low-Noise Readout Chain in CMOS Technology for APD-Based Small-Animal PET Imaging

[186]. Time-Mode Circuit Techniques for High-Performance Readout Interfaces

[188,192,193,204,206,408]. Review of Machine Learning for Real-Time Analysis at the Large Hadron Collider experiments ALICE, ATLAS, CMS and LHCb

[190]. 人工智能算法：推动未来的技术引擎

[194,197,213]. Real Time Machine Learning at the Large Hadron Collider

[195,198,211,478]. Development of artificial intelligence algorithms adapted to big data processing in embedded (FPGAs) trigger and data acquisition systems at the LHC

[196,212,479]. Real-time AI for Science

[200]. 下一代粒子对撞机实验中人工智能的应用

[202]. D5.1 Representative use cases: analysis and alignment

[203,205]. 人工智能分析实验数据

[207]. Exploring AI Methods to Confront Two Major Challenges at the LHC

[208]. 新机器学习技术在大型强子对撞机数据处理中的应用

[209]. 人工智能或助大型強子對撞機一臂之力

[210]. 机器学习在粒子物理学领域的当前应用和未来前景

[214,216,220,222,223,224,225,226,233,353,354,355,356,367,368,443,444]. Performance of a front-end prototype ASIC for the ATLAS High Granularity timing detector

[215,221,234]. Test and simulation of a Cherenkov picosecond timing counter

[217,235,416]. CHANTI: a fast and efficient charged particle veto detector for the NA62 experiment at CERN

[218,230,418,427]. TORCH: a fast timing detector for low-momentum particle identification

[219,417]. TWEPP 2021 Topical Workshop on Electronics for Particle Physics Book of Abstracts

[229]. Upgrades of the ATLAS Muon Spectrometer Front-end Electronics for the High-Luminosity LHC

[231,415]. Performance of the Time-of-flight system at the BM@N experiment

[232,447]. 19TH INTERNATIONAL COSMIC RAY CONFERENCE

[236]. SAPHIR ARM 2026

[237,263]. Pattern Recognition on Neural Networks using FPGA

[238]. Sixth FAIRmat Users Meeting

[239,249,256]. Real-time and Resource-efficient Embedded Computer Vision Via Optimizing Lightweight CNNs for FPGA Acceleration

[240,241,245,257,261,264,265,266]. ON THE OPTIMAL DEPLOYMENT OF DEEP LEARNING NEURAL NETWORKS ON FIELD PROGRAMMABLE GATE ARRAYS

[242,251,461]. FPGA Based Deep Learning Accelerator for RF Applications: A Design Framework

[243,244,246,247,248,262,462]. Implementation of Neural Network on FPGA Using HLS

[250]. A Convolutional-Neural-Network-Processor Design with an FPGA-Based Resource-Multiplexing Architecture

[252]. Low Latency In-Situ Image Analytics with FPGA-based Quantized Convolutional Neural Network

[253,258]. FPGA-Based Deployment of Pretrained CNNs for Edge AI: A Transfer Learning Approach

[254]. An FPGA-Based 8 Bit and 16 Bit Hybrid Precision Frequency Domain CNN Accelerator Design

[255,320]. Accelerating Transformer Neural Networks on FPGAs for High Energy Physics Experiments

[259]. LUNA: LUT-Based Neural Architecture for Fast and Low-Cost Qubit Readout

[260]. DEPLOYMENT OF NEURAL NETWORKS THROUGH PYNQ

[277,283,286]. 实验粒子物理计算研讨会（2023）

[279,285]. United Nations Activities on Artificial Intelligence (AI) 2024

[287]. Pattern recognition for particle shower reconstruction: Exploring AI-based methods for calorimetric clustering at the CMS HGCAL

[288,450]. Quasiparticle-like photonic topological textures

[289,475]. Towards neuromorphic readout of Particle tracking Detectors

[[290]. List of CERN Publications ](https://cds.cern.ch/record/1974088/files/Annual Report 1999.pdf)

[294]. AI-Based Sensor Systems and Applications

[295]. Readings in AI 2023

[296]. Quantum Measurements with Superconducting Nanowire Single Photon Detectors

[298]. EL DETECTOR V0A DEL EXPERIMENTO ALICE-CERN

[299,302]. NEWFIRM linearity calibration and correction

[300]. Development of a GridPix readout for a TPC at the International Linear Collider

[301]. Low Area and Low Power Time-to-Digital Converter for Pixelated Detector with 6.9 ps RMS Timing Jitter

[303]. Research and Materials on Hardware implementation of Transformer Models

[304,317,318,319,321,322,333,335,336,337,338,464,465]. Sub-microsecond Transformers for Jet Tagging on FPGAs

[305,312,313,314,315,334,339,340]. Efficient Implementation of Transformer Inference via a Tiled-Based Architecture on an FPGA*

[306,308,310]. TrackCore-F: Tracking Transformer Synthesis for Low-Latency FPGA Deployment

[307,309,326,328,330]. Optimizing Transformer Models for Low-Latency Inference: Techniques, Architectures, and Code Implementations

[316]. A Case for Low Bitwidth Floating Point Arithmetic on FPGA for Transformer Based DNN Inference

[323]. High-Performance FPGA Acceleration for Transformer-Based Models

[324]. Automating Versatile Time-Series Analysis with Tiny Transformers on Embedded FPGAs

[329]. A Cost-Efficient FPGA Implementation of Tiny Transformer Model using Neural ODE

[341,344,410,481]. Intelligent Networking AI-Driven Optimization in 6g Communication Systems

[342,345,411,482]. Communications of HUAWEI RESEARCH

[343,347,351,483]. Explainable AI for 6G Use Cases: Technical Aspects and Research Challenges

[346]. Status Report to TSG

[348,350]. 今年是中国5G商用牌照发放6周年，6G未来将至，会展现怎样的新图景？

[357,366]. ALICE 3-RICH project

[358]. Front-End Electronics for Fast Timing

[359]. Status of BNL Front End ASIC Design

[360,445]. The Front-End Electronics of the HADES timing RPCs wall: design, development and performances analysis

[361,364,446,453]. NEURAL NETWORK ENHANCED INSTRUMENTATION FOR SCALABLE TIME-OF-FLIGHT PET

[362,365,448]. A Neural Network Based Calibration Technique for TI-ADCs with Derivative Information

[363]. Timing performance of the Timepix4 front-end

[369,371,372,382,383,468,470]. FPGA-Based Accelerator for Quantized CNNs: High-Throughput Edge Deployment with Optimized Resource Utilization

[370,373]. Efficiently Training CNN with FPGA

[374,375,466,469]. Low-Latency In Situ Image Analytics With FPGA-Based Quantized Convolutional Neural Network

[376,472]. HARDWARE ACCELERATION WITH FPGA BASED ELECTRONIC BOARDS FOR MACHINE LEARNING

[377,473]. StrikeWatch: Wrist-worn Gait Recognition with Compact Time-series Models on Low-power FPGAs

[378]. Towards FPGA-deployable real-time boosted top jet identification

[379]. RoadNet-RT: High Throughput CNN Architecture and SoC Design for Real-Time Road Segmentation

[380,390]. REQ-YOLO: A Resource-Aware, Efficient Quantization Framework for Object Detection on FPGAs

[381,387]. An FPGA-Based SiNW-FET Biosensing System for Real-Time Viral Detection: Hardware Amplification and 1D CNN for Adaptive Noise Reduction

[384,467]. A hardware-friendly logarithmic quantization method for CNNs and FPGA implementation

[385,388,463]. FPGA-Based Quantization of CNNs for Resource-Constrained Devices

[386,471,474]. Design of Convolutional Neural Network Processor Based on FPGA Resource Multiplexing Architecture

[389]. Characterizing Parameter Scaling with Quantization for Deployment of CNNs on Real-Time Systems

[391]. MATURITY OF THE MAX IV LABORATORY IN OPERATION AND PHASE II DEVELOPMENT

[392]. A.I. Assisted Experiment Control and Calibration

[393]. AI-Powered Assistant for Long-Term Access to RHIC Knowledge

[394,396,397,451,459]. Using AI to predict calibration constants for the central drift chamber in GlueX at Jefferson Lab

[395,452,460]. Jefferson Lab: Year in Review

[398,403,404,405]. ILC Cherenkov Detector: Photodetector Studies

[399,400,401]. Factors Influencing the Long-Term Stability of Electronic Tongue and Application of Improved Drift Correction Methods

[402]. Development of a dynamic beam stabilization system for a six-axis ultrashort pulsed laser robot

[406]. AI calibration and anti-drift technologies—powered by data modeling, dynamic compensation, and intelligent learning—break the traditional trade-off between precision and long-term stability

[442]. Sparsity in neural networks: the trade-off between operation count and complexity for embedded systems

以下来源未被直接引用

ADVANCED SENSING STRUCTURES FOR ELECTROMAGNETIC SECURITY AND BIO-SYSTEM

DISTRIBUTED TIME-CONSTANT IMPEDANCE RESPONSES INTERPRETED IN TERMS OF PHYSICALLY MEANINGFUL PROPERTIES

智能电子秤设计

Physical Review X：机器学习打破衍射极限

Real-Time Trigger and online Data Reduction based on Machine Learning Methods for Particle Detector Technology

Transactions on Electrical and Electronic Materials

A Bibliography of Publications in IEEE Computer, 2020-2029

LLE in Focus | Summer 2025 | Issue 6

AI4Research: A Survey of Artificial Intelligence for Scientific Research

Developing AI-Based Predictive Analytics for Electric Vehicle Charging Infrastructure: Utilizing Machine Learning Models for Demand Forecasting, Charging Station Optimization, and Energy Management

A Complete Bibliography of Publications in IEEE Spectrum (2020-2029)

Memòria del Departament d'Enginyeria Electrònica Curs 2023/2024

AI-Driven Optimization of Renewable Energy Systems: A Comprehensive Review

CSR强子探测器读出电子学方法研究

An attention-based convolutional neural network for brain tumor segmentation

Channel Attention Mechanism Design

MDNet: A Semantically and Visually Interpretable Medical Image Diagnosis Network

A convolutional neural network and a bidirectional long short-term memory (BiLSTM) combined structural nonlinear model updating method based on attention mechanism

MBSNet: A Novel Multi-Branch Medical Image Segmentation Network

Dual-Attention Domain-Adaptive Segmentation Network (DADASeg-Net) for Cross-Modality Medical Image Segmentation

Contextual attention network for medical image segmentation

Non-local networks and self-attention mechanisms for modeling visual cortical neurons

Multimodal neural methods in medical imaging

MMM 2023 Abstracts Book

전자공학회지 The Magazine of the IEIE

Readings in AI 2023

Scholar Articles Since 2020

The Convergence Of Artificial Intelligence And Electronics: A New Era Of Intelligent Systems

HKUST DeepTech Playbook 2023

Jui-Sheng Rayson Chou P.E. • Ph.D.

Quant Magazine: 2023's 物理学最大的进展

Recent Advancements in AI: A Comprehensive Overview

Technical Design Report of the Time of Flight System (TOF)

HighEnergyPhysics detectors for medical applications

AI in Medical Imaging and Image Processing—Volume II

Transforming medical imaging with Transformers? A comparative review of key properties, current progresses, and future perspectives

Artificial intelligence (AI) is revolutionizing medicine by automating tasks like image segmentation and pattern recognition

Edge AI in Medical Imaging: Transforming Real-Time Diagnostics

人工智能在新型电力系统优化调度控制中的应用

Upgraded readout electronics for the ATLAS LAr Calorimeter at the Phase I of LHC

The SALT—Readout ASIC for Silicon Strip Sensors of Upstream Tracker in the Upgraded LHCb Experiment

A next generation readout system and its application at the NMX instrument for ESS

Validation of FPGA computing acceleration for the LHCb Upgrade

RD53 pixel readout integrated circuits for ATLAS and CMS HL-LHC upgrades

China contributions to Upgrade of LHC experiments

Pulling the trigger on LHC electronics – CERN Courier

INVESTIGATIONS OF A TIMING SILICON PARTICLE DETECTOR

4-Dimensional Tracking with Ultra-Fast Silicon Detectors

BDF/SHiP Annual Report 2025

Next-Generation Electronics for the Read-Out of Micro-Pattern Gaseous Detectors

人工智能（AI）硬件有望彻底被颠覆，在计算速度和能效方面实现前所未有的改进

Background Rejection for the ALPHA-g Anti-Hydrogen Detectors

Journal of Instrumentation

THE SLAC MARK II UPGRADE DRIFT CHAMBER FRONT END ELECTRONICS

AHCAL Front-end Electronics and DAQ

Choose and utilize metrics

A COST-EFFICIENT FPGA-BASED CNN-TRANSFORMER USING NEURAL ODE

FPGA Accelerator for Quantized Neural Networks

FAST READOUT ALGORITHM FOR CYLINDRICAL BEAM POSITION MONITORS PROVIDING GOOD ACCURACY FOR PARTICLE BUNCHES WITH LARGE OFFSETS

A Lightweight Transformer Model using Neural ODE for FPGAs

Low Latency Transformer Inference on FPGAs for Physics Applications with hls4ml

Topic Summary for [116bis][107] 6G AI

The Integration of AI and 6G: Exploring AI-Native air interfaces, use cases, and validation techniques for enhanced network performance

6G Test and Optimization

XGMF 6G-AI WG : AI × Communications for 6G

面向AI服务质量保障的6G无线网络智慧内生架构

ETSI 6G MONDAY CONFERENCE EURECOM Key 6G Research Axes

NTT技術ジャーナル Vol.37 No.7

Hexa-X: WP4 - Deliverable D4.3 AI-driven communication & computation co-design: initial solutions

A 410 GFLOP/s, 64 RISC-V Cores, 204.8 GBps Shared-Memory Cluster in 12 nm FinFET with Systolic Execution Support for Efficient B5G/6G AI-Enhanced O-RAN

WF on [117][107] 6G AI

Moderator summary #2 on AI/ML for 6GR

6G研究：发散创新的光芒

全球工程前沿 2020

Development of a Generic, Low-power and Multi-channel Frontend Readout ASIC for Precision Timing Measurements at EIC

Study and Design of ASIC Based Frontend Readout Electronics Topologies for High Energy Physics Experiments

This paper presents the design and characterisation of a front-end prototype ASIC for the ATLAS High Granularity Timing Detector

A Novel CNN Based Standalone Detector for Faster-than-Nyquist Signaling